# Suitability Analysis with Raster Data

# I.  Overview

A.  Objective:  This exercise will teach you to work with raster Layers using ArcGIS Spatial Analyst.  You will be creating and converting grid Layers to prepare them for input into an analysis model.  Then you will run a model to determine potential sites for a new landfill.

B.  Scenario:  The County needs a new landfill.  They want to use GIS to help locate suitable sites for the landfill. These sites will be based on characteristics of the land, water and soils around the county.   Before this analysis can be performed, however, the data must be prepared. (Note that this is a fictional scenario.)

C.  What you will need

- This exercise requires ArcGIS 8.2 (ArcView, ArcEditor or ArcInfo) and a license of the Spatial Analyst Extension.

- You will need to download the data from <INSERT URL HERE>.  Unzip the folder into a directory on your computer that will be referred to as <YourDirectory> in this exercise.

- Within this folder there will be a folder called lf_analysis, and within this folder are several other folders.  You should look at the contents of these directories in both ArcCatalog and Windows explorer before starting the exercise.  There is also a folder called Results that contains the results of all the analysis steps below.

- Start an ArcMap session and open landfill_analysis.mxd in <YourDirectory>/lf_analysis/.

D.  Notes on working with Grids and Spatial Analyst.

- Grids are actually collections of files within several folders in a directory.  Never move, rename, copy, delete or otherwise alter grids in windows explorer.  Always use ArcCatalog. .

- Grid names must be under 11 characters.  The name of a folder, grid, shapefile, table, or other file or folder should never contain spaces or ANY character other than a letter, number, or underscore_.

# E.  About the data

These data are from the Montana Natural Resources Information System (NRIS).  All are public-access data; however, they have been modified for the purposes of this exercise.  You should not use them for anything besides this exercise.  If you wish to acquire real data, you can go to the NRIS Web Site: The site may also be useful if you wish to include other factors in your analysis, such as wilderness areas, population centers, etc.

# II.  Define Your Study Area Using the Analysis Options and Make a Slope Map

A.  Enable the Spatial Analyst Extension.  To run this exercise, you will need the Spatial Analyst extension turned on.  This is a two-step process.

- Under the Tools menu, click Extensions... then check the Spatial Analyst box.

- Under the View menu, click Toolbars and check Spatial Analyst.  The Spatial Analyst toolbar will appear.  You may leave it floating or else dock it somewhere.

B.  Set Analysis properties.  When you do raster analysis, you will create a lot of new grids.  Each time you do, you must specify its cell size and map extent.  To save the effort of doing this every time and maintain consistency in the data that you create, you can set default cell size and extent.  This is done in the analysis options dialog.

- Spatial Analyst  Options.

- On the General tab, set your working directory to <YourDirectory>/lf_analysis/analysis.  Select county_bnd as the analysis mask.  Read the Analysis Coordinate System section but do not change anything.

- On the Extent Tab, select Same as Layer county_bnd from the pull-down.

- On the Cell Size tab, Select “As Specified Below” from the pull-down, then enter 1000 in the box below.

- Close the Analysis Options and Save your map document (FileSave or Ctrl-S).

C.  Make a slope map.  The elevation Layer is a surface of values are across an area.  From this surface of values we can derive other information, including slope, which indicates the rate of change, hillshading and contour lines.  Slope will be a relevant factor in siting a landfill, since the topography of an area is an important consideration for excavation.

- Make the elevation Layer visible.

- Spatial AnalystSurface Analysis Slope…

- Set the parameters as shown here (Fig. slope.jpg).  Input surface: Elevation.  Output measurement: Percent.  Accept the rest of the defaults (Z Factor 1, Output Cell Size 1000,  Output raster: <Temporary>).

- A new Layer will be created.  Its values should range from about 0.07… to 28.51…, and it should have the same shape as the county outline.  This is because of the analysis mask you set.

- Right-click on the Slope of Elevation Layer  Properties Source Tab.  Notice that this grid Layer is floating point/continuous, temporary dataset.   Explore the other tabs in this grid’s properties, then Close the Properties dialog.

- Since we want to keep this grid for future analysis, we will convert this temporary grid to a permanent one.  If it looks like it should (as described above), then Right-click  Layer  Save data set.  Save it as <YourDirectory>/lf_analysis/d_analysis/slopemap.

Analysis Extent vs. Analysis Mask.  The analysis extent sets the x-y coordinates for all the grid operations that you do.  The extent is always a box shape, regardless of whether you use another layer, the intersection or union of your inputs, or enter the x-y coordinates yourself.  An analysis mask works in much the same way as the analysis extents, except that it requires a layer to use as the mask, and it uses the actual shape of this layer, rather than just the rectilinear envelope.

To further illustrate this concept, go back to the Analysis Options (Spatial AnalystOptionsGeneral tab) and change the analysis mask to None.  Then run the slope and hillshade operations as described above.  Notice how the shape of the output grid looks different without an analysis mask set: the extent of the grid is the same as the County’s bounding box, but it does not honor the actual shape of the county.  After experimenting, change the analysis mask back to county_bnd and delete the temporary rasters you just made.

III.  Make grids from Feature Classes. For Spatial Analyst to use a layer in analysis, it must be converted to a grid.  Feature Classes (geodatabase or shapefile) and grids, as we’ve seen, have different data structures.

## A.  Make sure you have all of the data that you need for this analysis.   Your Data Frame should have the following layers in it, in this order in the TOC:

| #### Layer | Visible | Legend |
| --- | --- | --- |
| Landfills | no | show |
| Streams | yes | show |
| county_bnd | no | hide |
| Elevation | no | hide |
| Slope | no | hide |
| Coverclass | no | hide |

B.  Buffer Streams.  The distance from streams will also be a factor for siting  a landfill.  Although many of the existing facilities were built right on riverbanks, this is a less acceptable practice than it used to be.  Any potential landfill site will have to consider its distance from a water way.

- Make the streams layer active.

- Tools Buffer Wizard.  Use the parameters below to build the buffers; Click the Help… button if you need full instructions.

- First screen: Buffer the selected features of streams.

- Second screen: You want to make 4 multiple rings with 1000 meters, or 1 kilometer, between them.

- Third screen: Yes, dissolve barriers between buffers.  Save the buffers as a new layer (shapefile):<YourDirectory>/lf_analysis/d_analysis/Buffer_of_streams.shp.

- Be patient; the buffering may take a few minutes.

C.  Make a stream buffer grid

- The buffer layer will be added to your map.  Examine the Layer’s attribute table.  What fields are there?  How many records are there?  Close the table, and make the layer invisible.

- Spatial Analyst Convert… Features to Raster.  Select Buffer_of_streams.shp. as the input layer and ToBufDist as the field for cell values.  Name this grid <YourDirectory>/lf_analysis/d_analysis/buff_temp.  OK.

- A new grid is added.  It should be a floating/continuous grid (confirm this by opening the Layer propertiesSource tab). Notice that the county_bnd layer defines the shape of this grid, not the streams or the buffer shapefile that you just made.  This is because of the analysis mask you set previously. (Fig.:streambuff.jpg)

D.    Convert this to an integer grid .  This is a floating point grid, which will cause us problems later in the analysis, so we’ll change it to an integer grid.

- Spatial Analyst Raster calculator.  Type the following statement: int ([Buff_temp]).  Make sure there is a space between int and the parentheses.

- A new grid is added.  If it looks right,  Right-click on the Layer Make Permanent.  Save it as <YourDirectory>/lf_analysis/d_analysis/strmbuff.

- Change the Layer name to Stream Buffers.  Make it invisible.

- Delete buff_temp and buffer_of_streams.shp from your TOC and save your map now, if you haven’t lately.

E.  Add a grid of soil drainage. Soils are an important characteristic for landfill sites because they often indicate wetlands, rock depth and drainage.

- Add the Layer <YourDirectory>/lf_analysis/d_analysis/soildrain.lyr to your view.  This is a grid that has already been converted from a polygon feature class.

- Optional: look at the original polygon feature dataset in <YourDirectory>\lf_analysis\d_gallatin\county_data.mdb\soils

F.  Calculate distance from open landfills.  One thing that will affect where the new landfill will be is where current landfills are now.  You will use Spatial Analyst to calculate the distance of every cell in the county from the nearest open landfill.

- Move landfills to the top of your table of contents and make it visible.

- We are only interested in open landfills.  Selection Select by Attributes.   Set Landfills as the Layer, then type this statement into the query builder: "STATUS" = 'O'.  Two landfills in Gallatin County and one outside should be selected.

- Spatial Analyst  Distance  Straight line.   Set Landfills as the Distance To: Layer, and accept the rest of the defaults.  You do not need to set a maximum distance because we have an analysis mask to define the search area.

- A new raster layer is created.  Move the new grid below the landfill layer so you can see both.  The new layer should be the shape of the County and have values ranging from 0 to ~136,297…  These numbers represent meters from any given point to the nearest open landfill.

- If the layer looks like it should, save it to disk by Right-click on the Layer Make Permanent.  Name it <YourDirectory>/lf_analysis/d_analysis/sl_dist (for “straight-line distance”).

- Save your Map Document now.

# G.  Optional Challenge Step: Use slope as the cost surface for distance.

# Perform a distance as above, but select Slope of Elevation as the cost raster.  Notice how this creates a different surface from straight-line distance.  Try moving it below the hillshade to get a picture of the two are related.  You can either save this by making it permanent or delete it.

V.  Overview the Model.

A.   Design the model:  This has been mostly done for you.  This involves determining the relevant factors, getting data for these factors, making the data usable and documenting the entire process in a flow chart.

B.  Process the model: You will re-classify each of the 5 suitability factors on a scale of 1-5 to make a new integer grid for each factor.  Then you’ll combine all of these factors to get a ranking of each cell for landfill suitability, and finally  Re-classify this final grid from 1-5.

## C.  Prepare your project for analysis.

## Re-name the Data Frame “Suitability Analysis”

## Delete these Layers from your Data Frame: soils, streams, elevation.

- Right-click on Data Frame nameNew Group Layer.  Call it “Final Proposal.”

- Drag the following Layers into the “Final Proposal” Group Layer:  Landfills, county_bnd, hillshade.  Make this group layer invisible for now.

## Activate “Suitability Analysis.”  Your Data Frame should have the following Layers in it, in this order in the TOC:

| #### Layer | Visible | Legend | Purpose |
| --- | --- | --- | --- |
| Sl_dist | yes | Hide | Factor in suitability analysis |
| soildrain | yes | Hide | Factor in suitability analysis |
| strmbuff | yes | Hide | Factor in suitability analysis |
| Coverclass | no | Hide | Factor in suitability analysis |
| Slope | no | Hide | Factor in suitability analysis |
| Final Proposal | no | Hide | Group layer of background data |

# VI.  Reclassify all the Layers

A.  Overview.  You will reclassify each layer on a scale of 1 to 5, with 5 being most suitable  for a new landfill and 1 being least.  These are subjective rankings; in the real-world of suitability analysis, these factors could be ranked based on many factors including public input, scientific or economic data, and others.  For the purpose of this exercise, the subjective values for most factors have been decided, but you could try changing them to meet your own criteria.

You can also exclude a category from analysis by re-classifying it as NoData.  Below is the general process used in re-classifying a raster Layer.  The specifics for each Layer will be detailed in the following steps (B-E).

- To reclassify a Grid, use the Spatial AnalystReclassify… dialog.  This dialog works a lot like the Symbology properties.  You pick a value to classify on, then assign a classification method and the number of classes.

- You will use different methods to re-classify each Layer, but all will produce the same effect: rating the values of each grid on a scale of 1-5, with 5 being most suitable for a new landfill and 1 being least.

- A new temporary grid will be created. You will then make this temporary raster permanent.

- It will be easier to examine the new grid if you change each of the legends to a Graduated Color legend with 5 classifications based on value. Use a different color ramp for each re-class Layer.

B.  Classify the slope Layer explicitly.  This is how all re-classifications begin: you will enter the old values and new values manually.

- Spatial Analyst Reclassify....  Make Slope of Elevation the Input Raster.

- Click the Classify… button to enter the classification dialog.

- Change the Classification Method to Equal Interval, then specify 5 classes.  OK.

- Change the values as follows (Old ValueNew Value)

- 0 - 12; 1 - 25; 2 - 54; 5 - 123; 12 - 301; No DataNo Data.

- This is easiest if you change all of the values in the Old Values column first, then change all of the values in the New Value column.  Important: you must have a space between each value and the dash between them in the Old Value column.

- A new grid is created.  Change the legend to Classified with 5 classes.

- If the new Grid looks like it should, Right-click on the LayerMake Permanent… Save it as <YourDirectory>/lf_analysis/d_reclass/slope_r1

- Notice that there is not a linear relationship between slope and suitability. Flat areas are less suitable than rolling areas, because flat land in western Montana can be put to better use than storing garbage.  However, very steep areas are unsuitable because of problems associated with excavations on steep slopes.

C.  Reclassify Distance to Open Landfills from a Saved Classification.  The same is true of distance and suitability.  The least suitable sites are those closest to open landfills; however, the next least suitable sites are those farthest away.  This reflects a subjective value: we don’t want landfills right on top of each other, but we don’t want them too far away, as this could cause management and maintenance headaches.

- Spatial Analyst  Reclassify....  Make the distance to landfills Layer the Input Raster.

- Click Load at the bottom of the box.  Load the file <YourDirectory>/lf_analysis/tables/reclass_distance.  OK.

- A new grid is created.  Change the legend to a graduated color with 5 classes.

- If the new Grid looks like it should, Right-click on the LayerMake Permanent… as <YourDirectory>/lf_analysis/d_reclass/distance_r1.

D.  Add a reclassification grid for soil drainage.

- The soil drainage has already been re-classified for you by the county geologist.  Add the layer <YourDirectory>\lf_analysis\d_reclass\soildrain_r1.

- Notice that this layer prohibits landfills on water and rock by re-reclassifying hem as NoData..

E.  Re-classify stream buffers by Unique Value.

- Spatial Analyst  Reclassify.... Make the strmbuf Layer the Input Raster.

- Click the Unique button at the top.

- Accept the default values, except for the NoData.  Change the New Value for NoData to 5.  This is because the areas more than 4km away from a stream are the most suitable for landfills, while areas close to streams are less suitable.

- OK.  A new grid is created.  Change the legend to Clasified with 5 classes

- If the new Grid looks like it should, Right-click on the LayerMake Permanent…c:/geogcp/ex16model/d_reclass/buffers_r1.

F. Re-classify land cover with your own classification

- You will determine the classification for the final suitability factor, land cover class.  Remember that you will be actually re-classifying the value, which is an arbitrary number assigned to the more meaningful s_value.

- Make the coverclass layer visible in your TOC; move it to the top or turn off layers above it so you can see it.

- Use this table to write out your classification scheme before entering it in ArcMap.  Some things to consider are the cultural or financial value of the existing land type, the expense of altering the land type, and the relative abundance or scarcity of the land type (the count field).

| Old Value | New Value | S_VALUE | COUNT | Reason |
| --- | --- | --- | --- | --- |
| 1 |  | Deciduous/Mixed Forest | 3739 |  |
| 2 |  | Coniferous Forest | 11904 |  |
| 3 |  | Cropland | 2974 |  |
| 4 |  | Grassland | 4880 |  |
| 5 |  | Desert Shrub and Savanna | 2157 |  |
| 6 |  | Water | 27 |  |
| 7 |  | Alpine, Tundra, Barren | 169 |  |

- Spatial Analyst  Reclassify.... Make coverclass the Input Raster.

- Click the Unique button at the top.

- Enter your classification above and click OK.

- A new grid is created.  Change the legend to a grad color with 5 classes.

- If the new Grid looks like it should, LayerMake Permanent… as c:/geogcp/ex16model/d_reclass/cover_r1

## VII.  Run the model  Before starting, make a new text file in <YourDirectory>/lf_analysis called landfillmodel.txt

A. Run an arithmetic model.  In this type of model, each of the grids has equal weight in the decision-making process.

- Spatial Analyst  Raster Calculator

- On the left is a list of all the grids in your Data Frame.  The box is too narrow to show the complete name, so you’ll need to scroll to see the entire list of grids and their full names.

- The map calculator works a lot like the query builder.  Double-click on the grid names and the operators to build the following statement [reclass of coverclass] + [reclass of sl_dist] + [Reclass of Slope of elevation] + [Reclass of soildrain] + [reclass of strmbuff].  Just like in the query builder, exact syntax is critical.  Spacing, parentheses and operators must be the same.

- Copy this text and paste it into the text file.  Annotate this with something like: “First arithmetic model”.

- Click Evaluate.  A new temporary grid will be created. Notice that the range of values is between 7 and 25.

- Change the symbology to Classified with a single class for each value. (Fig.: model_a25.jpg)

- If the new Grid looks like it should, save your data set as <YourDirectory>/lf_analysis/d_analysis/model_a25.

- In the text file, write some information about this model: what data went into it, purpose of the analysis, cell size and units, extent (Gallatin Co. Montana), when you ran it, and what organization you did it for.

- Optional: Re-classify this final output  (Spatial AnalystReclassify…).  You may just divide it up equally, or you may want to tweak the classification; for example, only sites scoring a 24 or 25 fall into the most suitable category.  Save this grid as ex15model/d_model/ model_a5.   Document how you divided the classes up in your text file.

C: Optional: Re-run the model using weighted values.  In the real world, certain features are more relevant than others in a suitability model.  Next, try running a model in which slope and soils have the most influence.

- The syntax will be slightly different than the arithmetic model.  Each grid will have a decimal weight attached to it; the total of the decimals must add up to 1.  Note that there are extra parentheses.  You can use these weights, or use weights of your own design.

- (( [Reclass of soildrain]*0.3) +( [Reclass of Slope…] * 0.3) + ([Reclass of Strmbuff]*0.2) +( [Reclass of coverclass]*0.1) + Reclass of Distance to Landfills]*0.1))*5   We are multiplying by 5 because that is the total number of factors in the equation

- Copy this text and paste it into the text file.  Annotate this with something like: “First weighted model”.

- Click Evaluate.  A new temporary grid will be created.

- If the new Grid looks like it should, save your data set ex15model/d_model/ model_w25.

VIII.  Further Analysis and Presentation of Results

This exercise will be continued.  In the next session, you will use this analysis to select potential areas for the new landfill and to present your findings in a series of maps.  There will also be resources for performing this analysis at a finer scale than the 1km resolution of this exercise.